1、Unzip the file, correct upgrade file name is suffixed with “web”

2、Assigned manually IP address 192.168.0.x （“x” range from 2 to 253）, the Subnet Mask is 255.255.255.0 and Gateway is 192.168.0.254.

3、Enter our web interface，192.168.0.254

4、Login，the username and password are admin

5、Upgrade transition version: CP450-1C-1H-Transitional version

Management->upgrade firmware, click ②, pop up a Window, ③Please select the correct file, ④finally click "Upgrade". Then wait a few minutes, then the CPE automatically restarts.

[ Notice ]

DO NOT power off the device or close the browser window during the upload because it may crash the system.

6、Upgrade stable version: V4.1.0cu.747_B20191224_ALL

Log in to the new CPE page and upgrade the firmware again. Then wait a few minutes, then the CPE automatically restarts.

[ Notice ]

DO NOT power off the device or close the browser window during the upload because it may crash the system.

7、After the upgrade is successful, must be reset the CPE.

[Attention ]

- CP450-1C-1H-Transitional version is an informal version, only for upgrade transitions.

- Please strictly follow the upgrade instructions in the zip package.

If you encounter any problems during the upgrade process, please contact your local after-sales staff or email to "fae@zioncom.net".

Q1：After the upgrade is successful, you cannot log in to the CPE page, which may be related to factors such as CPEs and browsers. Please reset the CPE and clean up the browser cache.

- 1.  Reset CPE：

Please make sure your CPE’s power is on regularly, then press the RST button for about 5~8s. Loosen the button until your CPE’s LED lights all flashing, then you have reset your CPE to default settings.

2. Clear your browser cache：

Delete cookies on the web browser. Here we take Firefox browser for example.

Note: In general, the browser enters the management address of the CPE and the page display is incomplete. Please use this method first.